Tom Jennings

11 Oct 88

This is how the Zmodem implementation in Fido/FidoNet 12i and

FidoTerm version 2 works.

This is a fully compatible, standard Zmodem implementation, with

a few fancy features added. You can, and probably should adjust

Zmodems behaviour with the two controls in FIDO.INI (details

follow), because Zmodem can potentially accept data faster than

your computer can handle. The default settings are quite

conservative, and should work on all machines.

Zmodem Features

This Zmodem implementation has the following basic

characteristics: (Skip this if you don't care)

Block sizes:	1024 bytes 	above 2400 baud

512		2400 baud

256		1200 baud and below

Upon four consecutive errors on the same block, the block size is

halved; minimum block size is 64 bytes.

Upon forty consecutive blocks with no errors and no line noise

junk characters, block size is doubled; maximum block size is

1024 bytes.

Zmodem Controls

Zmodem Receive controls allow either full streaming, or fully

acknowledged blocks.

Zmodem Transmit controls allow full streaming, or a sliding

window, the window size stated in either Kbytes or number of

blocks.

Zmodem Installation

PLEASE: You have to very thoroughly TEST, TEST, TEST after you

change Zmodem settings, particularly if you enable full

streaming. Especially at 9600 baud and above, wrong settings can

make the difference between breathtaking 1100 byte per second

transfers, and performance worse than the slowest XMODEM. Make

sure your computer can actually handle what you tell Zmodem to

do.

Keep in mind the whole point of having high speed modems and

protocols was so that you can run as fast as your machine allows;

a modem capable of 1500 characters per second doesn't make your

computer any faster, all it guarentees is that it won't hold you

back.

Computers and Modems

I don't have enough experience with Zmodem at this point to make

hard reccomendations. Zmodem performance is limited by the modem

and computer (processor and disks) combination you have, plus

other factors like multitaskers and LANs. I'll just list some

anecdotal stuff:

If you have a very fast modem (US Robotics HST, Telebit

Trailblazer, Hayes V-series, etc) and an ordinary pclone low or

medium performance computer, you will have to be very careful

with your installation; it is extremely easy to make Zmodem

accept data faster than your computer can write it to disk!

If you have a 300/1200 modem, use full streaming for receive, and

a sliding window of 4 to 8 blocks; start with 6. At 1200 baud,

you'd be hard pressed to have Zmodem deliver data to your

computer faster than it could write it to disk.

If you have funny background software like LANs, multitaskers

like DoubleDOS, DESQview, TOPView, etc, certain popup utilities,

funny device drivers, you may find that they slow your computer

down, when they are active, and make full streaming fail

miserably.

High performance machines like fast 286's or 386 boxes can

probably run full streaming, probably even with multitaskers.

If you've got a "turbo" pclone with the usual slow as molasses

disk drive, and a fast modem, you can probably make full

streaming work if you don't have any funny software running.

For example, the Fido Software BBS is a cheap "turbo" pclone

running at 8MHz, with an 80 mS disk drive, and a US Robotics HST

locked at 9600 baud (modem type 16). This seems to work just

fine, it accepts uploads from a PS/2 and HST locked at 19,200

baud at full speed without a hitch.

But, with Fido set for modem-type 17 (locked at 19,200) instead

of modem-type 16 (locked at 9600) it fails miserably. Take this

as a hint as to what you're getting into.

HINT: Always use a sliding window, even if it's huge, and huge is

not better than small. Start with a tiny window, make it slightly

larger, and when an increase in window size doesn't affect

performance, use the next size smaller.

All that a large window size will do for you is make for TERRIBLE

ERROR RECOVERY!!! Don't overdo it!

HINT: Don't look at the Senders modem activity lights when

adjusting window size; look only at the Receivers lights. The

senders activity can be misleading; for example, the US Robotics

HST has a 32K byte internal buffer, so Zmodem fills it quickly

then sits and waits for window synchronization; don't let this

fool you into thinking you could make it faster, you can't. Data

can only flow out of the modem into the phone line as fast as it

goes, all that increasing the window size will do is make error

recovery slower, and will slow down the transfer incredibly.

Modem Installation

Zmodem makes modem installation critical if you have a "9600

baud" modem. Most of these modems use a "locked baud rate"

between the modem and computer; what this means is that

regardless of the speed that your BBS modem connects to a caller

or other BBS, the computer and modem communicate at a fixed baud

rate, usually 9600 or 19,200 baud. This is done for performance

reasons, plus the related fact that "9600 baud" modems don't work

exactly the same as 2400 and below modems, and there is this

"hardware handshake" junk that goes on between the modem and

computer so that they can avoid losing data due to the high data

rate.

9600 baud is quick stuff; about 960 characters a second, and at

one interrupt per character the poor processor is hard pressed to

keep up with the data flow. Slow machines lose data and get lots

of errors; fast ones make it.

If you've got a machine less than 8MHz, especially if it's an

8088, use the variable rate modem type or locked at 9600. Locked

at 19,200 worked OK with Xmodem, because the blocks were so

small, just as the computer was about to lose it the data stopped

coming in and it caught up; with Zmodem it won't get the chance.

Zmodem Controls

The receive controls affect only how your Fido/FidoNet or FT

program receives files; if someone else calls in to download

files, Zmodem will go as fast as their Zmodem tells Fido or FT to

go. (They may have done something like this on their end as

well.)

---------------- RECEIVE: FULL STREAMING ----------------

FT:		0/D

Fido: 		zm-rx-type 0

When receiving file(s), tells the sending program that it can

accept data at maximum possible data rate, ie. full streaming.

This is meant for reasonably high performance machines that can

accept data at "high speed", whatever that means to you. It will

probably get lots of errors if you have a multitasker running, or

have really noisy phone lines, or a slow machine, etc.

---------------- RECEIVE: FULLY ACK'ED ----------------

FT:		1/D

Fido:		zm-rx-type 1

When receiving files, every block will be acknowledged. For

sending, Fido/FT will do whatever the receiver says, of course.

If for instance your 4.77MHz pclone with an 80 mS drive Fido is

running under DoubleDOS, and a caller calls in with a 20MHz 386,

and downloads a file to a RAMdisk, Fido will only send the data

as fast as it possibly can; if that same hardware hog were to

upload however, Fido would still insist on ACKing every block.

Note that regardless of the senders hardware, you would be hard

pressed to overrun even a lowly 4.77MHz Fido at 2400 baud or

below. (Well, maybe at 2400 ... you'll have to try it.) No

processor in the world can push more data through a slow modem.

------------ TRANSMIT: VARIABLE SLIDING WINDOW ------------

FT:		<blocks>/U		1 - 64

Fido:		zm-tx-type <blocks>	1 - 64

This is the preferred method of defining a sliding window. When

sending files, and the receiver says it can accept full streaming

Fido/FT will send data in full streaming mode, as long as it

receives acknowledges from the receiver every so many <blocks>.

The receiver sends occasional acknowledges, and the sender checks

for them, without pausing the data flow. If the sender doesn't

see an acknowledge it will stop and wait for one.

In most cases, this has all the speed of full streaming, with

much improved error recovery. The slight penalty is the reverse

channel does get used, which could slow some > 2400 modems down.

Certainly at 2400 and under there seems to be no penalty at all.

This will be the default mode for Fido and FT, I think.

Since the window size is stated in blocks, the size of the window

depends on the baud rate and error rate; if many errors occur,

Zmodem shrinks the block size, and hence the window shrinks too;

if the error rate is exceptionally good, the block size increases

as Zmodem increases block size. Higher baud rates start with

larger blocks.

Window size is:

window size = <blocks> * block size

I'd recommend starting with <blocks> at 6, which works out to be

a 1.5K byte window at 300 and 1200 baud, 3K at 2400, and 6K at

9600 and beyond.

HINT: Don't look at the Senders modem activity lights when

adjusting window size; look only at the Receivers lights. The

senders activity can be misleading; for example, the US Robotics

HST has a 32K byte internal buffer, so Zmodem fills it quickly

then sits and waits for window synchronization; don't let this

fool you into thinking you could make it faster, you can't. Data

can only flow out of the modem into the phone line as fast as it

goes, all that increasing the window size will do is make error

recovery slower, and will slow down the transfer incredibly.

------------ TRANSMIT: FIXED SIZE SLIDING WINDOW ------------

FT:		<windowsize>/U

Fido:		zm-tx-type <windowsize> 1024 - 65536

This is the second method of defining a sliding window. It works

the same as the previous method, except the size of the window is

fixed, and specified in Kbytes. An 8K window is an 8K window,

whether it contains 8 1024 byte blocks or 32 256 byte blocks.

The variable size sliding window is probably more flexible, in

that if the block size decreases due to line noise or whatever,

the window size decreases too.